Course 35 ---- DS1302 and LCD1602

The purpose of the experiment:

In this course we mainly study the use of DS1302 module

The actual object is shown below.

List of components required for the experiment:

Arduino UNO board *1

USB cable *1

DS1302 module  *1

Breadboard *1

Dupont line *1 bunch

Actual object connection diagram：

We need to connect the circuit as shown in the figure below.

Experimental code analysis:

- /**

- * @par Copyright (C): 2010-2019, Shenzhen Yahboom Tech

- * @file         35.DS1302 Serial port display time

- * @author      liusen

- * @version      V1.0

- * @date         2017.11.14

- * @brief        Arduino connect to DS1302

- Serial input: 2017,11,10,11,59,20,6

- Note:     1-7 Sunday - Saturday

- * @details

- * @par History

- *

- */

- #include <stdio.h>

- #include <string.h>

- #include <DS1302.h>

- uint8_t CE_PIN   = 5;

- uint8_t IO_PIN   = 6;

- uint8_t SCLK_PIN = 7;

- char buf[50];

- char day[10];

- String comdata = "";

- int numdata[7] ={0}, j = 0, mark = 0;

- DS1302 rtc(CE_PIN, IO_PIN, SCLK_PIN);

- /**

- * Function       print_time

- * @author        liusen

- * @date          2017.11.14

- * @brief         Serial printing time

- * @param[in]     void

- * @retval        void

- * @par History

- */

- void print_time()

- {

- Time t = rtc.time();

- memset(day, 0, sizeof(day));

- switch (t.day)

- {

- case 1: strcpy(day, "Sunday"); break;

- case 2: strcpy(day, "Monday"); break;

- case 3: strcpy(day, "Tuesday"); break;

- case 4: strcpy(day, "Wednesday"); break;

- case 5: strcpy(day, "Thursday"); break;

- case 6: strcpy(day, "Friday"); break;

- case 7: strcpy(day, "Saturday"); break;

- }

- snprintf(buf, sizeof(buf), "%s %04d-%02d-%02d %02d:%02d:%02d", day, t.yr, t.mon, t.date, t.hr, t.min, t.sec);

- Serial.println(buf);

- }

- /**

- * Function       setup

- * @author        liusen

- * @date          2017.11.14

- * @brief         Initialization configure

- * @param[in]     void

- * @retval        void

- * @par History

- */

- void setup()

- {

- Serial.begin(9600);

- rtc.write_protect(false);//turn off write protection

- rtc.halt(false);

- }

- /**

- * Function       loop

- * @author        liusen

- * @date          2017.11.14

- * @brief         Serial port timing and display

- * @param[in]     void

- * @retval        void

- * @par History

- */

- void loop()

- {

- while (Serial.available() > 0)

- {

- comdata += char(Serial.read());

- delay(2);

- mark = 1;

- }

- if(mark == 1)

- {

- Serial.print("You inputed : ");

- Serial.println(comdata);

- for(int i = 0; i < comdata.length() ; i++)

- {

- if(comdata[i] == ',' || comdata[i] == 0x10 || comdata[i] == 0x13)

- {

- j++;

- }

- else

- {

- numdata[j] = numdata[j] * 10 + (comdata[i] - '0');

- }

- }

- Time t(numdata[0], numdata[1], numdata[2], numdata[3], numdata[4], numdata[5], numdata[6]);

- rtc.time(t);

- mark = 0;j=0;

- comdata = String("");

- for(int i = 0; i < 7 ; i++) numdata[i]=0;

- }

- print_time();

- delay(1000);

- }

- Experimental steps:

- We need to open the program for this experiment:

- code_DS1302.ino, click “√”under the menu bar,compile the program, and wait for the words of Done compiling in the lower left corner, as shown in the following figure.

- In the menu bar of Arduino IDE，you need to select the 【Tools】---【Port】--- select the port that the serial number displayed by the device manager just now.for example:COM6,as shown in the following figure.

- After the selection is completed, you need to click “→”under the menu bar,and upload the program to the Arduino UNO board, when appears to Done uploading on the lower left corner , that means that the program has been successfully uploaded to the Arduino  UNO  board, as shown in the following figure.

4.After the code is uploaded.You need to open the serial monitor of the Arduino IDE and print out the date and time of initialization. You can also send the current date and time through the serial port and print it out, as shown below.

(Note: The format of inputting the date and time, as shown in the following figure.)