Course 36 ---- DS1302 and LCD1602

The purpose of the experiment:

In this course we mainly study the use of DS1302 module and LCD1602 module.

The actual object is shown below.

List of components required for the experiment:

Arduino UNO board *1

USB cable *1

DS1302 module  *1

LCD1602 module *1

Adjustable resistor *1

Breadboard *1

Dupont line *1 bunch

Actual object connection diagram：

We need to connect the circuit as shown in the figure below.

Experimental code analysis:

- /**

- * @par Copyright (C): 2010-2019, Shenzhen Yahboom Tech

- * @file         36.DS1302 and LCD1602

- * @author       liusen

- * @version      V1.0

- * @date         2017.11.14

- * @brief        Arduino Sensor Kit

- * @details      Arduino connect to DS1302

- Serial input: 2017,11,10,11,59,20,6

- Note:     1-7 Sunday - Saturday

- * @par History

- *

- */

- #include <stdio.h>

- #include <string.h>

- #include <DS1302.h>

- #include <LiquidCrystal.h>

- const uint8_t CE_PIN   = A3;

- const uint8_t IO_PIN   = A4;

- const uint8_t SCLK_PIN = A5;

- char day[10];

- int numdata[7] ={0};

- DS1302 rtc(CE_PIN, IO_PIN, SCLK_PIN);

- LiquidCrystal lcd(12,11,10,5,4,3,2);

- /**

- * Function       print_time

- * @author        liusen

- * @date          2017.11.14

- * @brief         Print time to serial port

- * @param[in]     void

- * @retval        void

- * @par History

- */

- void print_time()

- {

- char buf[30];

- Time t = rtc.time();

- memset(day, 0, sizeof(day));

- switch (t.day)

- {

- case 1: strcpy(day, "Sunday"); break;

- case 2: strcpy(day, "Monday"); break;

- case 3: strcpy(day, "Tuesday"); break;

- case 4: strcpy(day, "Wednesday"); break;

- case 5: strcpy(day, "Thursday"); break;

- case 6: strcpy(day, "Friday"); break;

- case 7: strcpy(day, "Saturday"); break;

- }

- snprintf(buf, sizeof(buf), "%s %04d-%02d-%02d %02d:%02d:%02d", day, t.yr, t.mon, t.date, t.hr, t.min, t.sec);

- Serial.println(buf);

- }

- /**

- * Function       print_time

- * @author        liusen

- * @date          2017.11.14

- * @brief         LCD1602 display

- * @param[in]     void

- * @retval        void

- * @par History

- */

- void display_time()

- {

- char  temp[30] = {0};

- Time t = rtc.time();

- char week[5];

- switch (t.day)

- {

- case 1:  strcpy(week , "Sun"); break;

- case 2:  strcpy(week ,"Mon"); break;

- case 3:  strcpy(week ,"Tues"); break;

- case 4:  strcpy(week ,"Wed"); break;

- case 5:  strcpy(week ,"Thur"); break;

- case 6:  strcpy(week ,"Fri"); break;

- case 7:  strcpy(week ,"Sat"); break;

- }

- snprintf(temp, sizeof(temp), "%02d-%02d-%02d %s", t.yr, t.mon, t.date, week); //t.yr, t.mon, t.date, t.hr, t.min, t.sec ,week

- lcd.setCursor(1,0);

- lcd.print(temp);

- snprintf(temp, sizeof(temp), "%02d:%02d:%02d", t.hr, t.min, t.sec );

- lcd.setCursor(4,1);

- lcd.print(temp);

- }

- /**

- * Function       setup

- * @author        liusen

- * @date          2017.11.14

- * @brief         Initialization configure

- * @param[in]     void

- * @retval        void

- * @par History

- */

- void setup()

- {

- Serial.begin(9600);   //The baud rate is 9600

- rtc.write_protect(false);  //turn off write protection

- rtc.halt(false);

- lcd.begin(16, 2); //Define 1602 liquid crystal display range is 2 rows and 16 columns of characters

- lcd.home();  //Move the cursor back to the upper left corner, that is, output from the beginning

- lcd.clear();

- Time t(2017, 11, 25, 18, 39, 25, 7);

- rtc.time(t);

- rtc.write_protect(true); // turn on write protection

- }

- /**

- * Function       loop

- * @author        liusen

- * @date          2017.11.14

- * @brief         Print and display the current time

- * @param[in]     void

- * @retval        void

- * @par History

- */

- void loop()

- {

- print_time();

- display_time();

- delay(1000);

- }

- Experimental steps:

- We need to open the program for this experiment:

- code_DS1302_and_LCD1602.ino, click “√”under the menu bar,compile the program, and wait for the words of Done compiling in the lower left corner, as shown in the following figure.

- In the menu bar of Arduino IDE，you need to select the 【Tools】---【Port】--- select the port that the serial number displayed by the device manager just now.for example:COM6,as shown in the following figure.

- After the selection is completed, you need to click “→”under the menu bar,and upload the program to the Arduino UNO board, when appears to Done uploading on the lower left corner , that means that the program has been successfully uploaded to the Arduino  UNO  board, as shown in the following figure.

- We can see the current date and time on the LCD 1602, as shown in the following figure.

(Note: If the LCD 1602 cannot be displayed, you can adjust the contrast by turning the adjustable resistor. The 1602 will display normally)